## Product Datasheet

| ## Modbus Register Address and commands. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ## Address | ## Name | ## FC | ## Qty | ## Description | ## Values / Notes |
| ## 0 | ## POSX | ## 16 | ## — | ## Shape/bitmap X start | ## pixels |
| ## 1 | ## POSY | ## 16 | ## — | ## Shape/bitmap Y start | ## pixels |
| ## 2 | ## ENDX | ## 16 | ## — | ## Shape/bitmap X end | ## pixels |
| ## 3 | ## ENDY | ## 16 | ## — | ## Shape/bitmap Y end | ## pixels |
| ## 4 | ## SHAPE | ## 16 | ## — | ## Shape type | ## 0=RECT, 1=TRI, 2=CIRC, 3=SLINE, 4=LINE |
| ## 5 | ## SHAPE_Q | ## 16 | ## — | ## Shape quantity | ## count |
| ## 6 | ## SHAPE_WEIGHT | ## 16 | ## — | ## Shape line weight | ## pixels |
| ## 7 | ## SHAPE_COLOR | ## 16 | ## — | ## Shape color | ## RGB565 uint16 |
| ## 8 | ## BITMAP | ## 16 | ## — | ## Bitmap file number | ## index |
| ## 9 | ## BITMAP_Q | ## 16 | ## — | ## Bitmap quantity | ## count |
| ## 10 | ## BORDER_WEIGHT | ## 16 | ## — | ## Border line weight | ## pixels (0 = none) |
| ## 11 | ## BORDER_COLOR | ## 16 | ## — | ## Border color | ## RGB565 uint16 |
| ## 12 | ## TEXT_POSX | ## 16 | ## — | ## Text X position | ## pixels |
| ## 13 | ## TEXT_POSY | ## 16 | ## — | ## Text Y position | ## pixels |
| ## 14 | ## TEXT_COLOR | ## 16 | ## — | ## Text color | ## RGB565 uint16 |
| ## 15 | ## FONT_TYPE | ## 6 | ## 1 | ## Font selection | ## 0=Arial8x11, 1=Arial19, 2=ArialItalic11, 3=SegoeUI14 |
| ## 16 | ## SI_UNIT | ## 6 | ## 1 | ## SI unit suffix index | ## index into SI_ table |
| ## 17 | ## SI_DECIMAL | ## 6 | ## 1 | ## Decimal places | ## 0–3 |
| ## 18 | ## LO_LIMIT | ## 6 | ## 1 | ## Graph scale min | ## int16 |
| ## 19 | ## HI_LIMIT | ## 6 | ## 1 | ## Graph scale max | ## int16 |
| ## 20 | ## ALO_LIMIT | ## 6 | ## 1 | ## Alarm low threshold | ## int16 |
| ## 21 | ## AHI_LIMIT | ## 6 | ## 1 | ## Alarm high threshold | ## int16 |
| ## 22 | ## ALO_COLOR | ## 6 | ## 1 | ## Alarm low color | ## RGB565 uint16 |
| ## 23 | ## AHI_COLOR | ## 6 | ## 1 | ## Alarm high color | ## RGB565 uint16 |
| ## 24 | ## ROTDIR | ## 16 | ## — | ## Scroll direction | ## int16 |
| ## 25 | ## ROTSPEED | ## 16 | ## — | ## Scroll speed | ## int16 (0 = static) |
| ## 26 | ## FILL_SCREEN | ## 16 | ## — | ## Fill screen enable | ## int16 |
| ## 27 | ## FILL_COLOR | ## 16 | ## — | ## Fill screen color | ## RGB565 uint16 |
| ## 28 | ## TEXT_GAP | ## 6 | ## 1 | ## Marquee gap (px) | ## pixels between repeats |

## Basic Use Quick Notes and Standards

| Category | Specification |
| --- | --- |
| Model | LED_O |
| Dimensions | W 100 x H 428.5 x D 100 (mm) |
| Weight | 1.25 kg |
| Enclosure Material | ABS + Polycarbonate Diffuser |
| Ingress Protection | IP40 / IP54 (depending on installation) |
| Power Voltage | 24 V DC ±10% |
| Rated Current Consumption | 1.1 A |
| Power Protection | Reverse‑polarity, Over‑current, Over‑voltage |
| Display Type | RGB LED Array 32 × 124 |
| Pixel Pitch | 3 mm |
| LED Type | SMD2121 RGB |
| Color Output | 16‑bit (5‑4‑5) RGB |
| Brightness | 800–1200 cd/m² (after diffuser) |
| Viewing Angle | ≥140° (panel native ≥160°, limited by circular housing and diffuser) |
| Refresh Rate | ≥1920 Hz |
| Scan Type | 1/16 |
| Communication Interface | Ethernet (Modbus TCP) |
| Default IP Address | 192.168.1.50 |
| Supported Modbus Functions | FC03, FC06, FC16 |
| Max Connections | 1 TCP client |
| Response Time | <10 ms typical |
| Interface | PC (HTTPS) / PLC |
| Output | RGB LED Color |
| Operating Temperature | 0 °C to 60 °C |
| Storage Temperature | −40 °C to 85 °C |
| Relative Humidity | 30%–90% RH (non‑condensing) |
| Operating Height | Limited by viewing angle |
| Vibration | 10–50 Hz, 10 m/s² (1.0 G), 10 min |
| Vibration (Damage Limit) | 10–50 Hz, 20 m/s² (2.0 G), 2 h |
| Drop Test | 1 m (5 ft) onto concrete |
| LED Lifetime | >50,000 h (L70) |
| Firmware Version | v1.0.0‑R (Release) |
| Firmware Update Method | Ethernet (internal service tool) |
| Certifications | CE / UKCA |

| Ethernet (default) |  |
| --- | --- |
| IP Address | 10.0.0.49 |
| Subnet Mask | 255.255.255.0 |
| Gateway | 10.0.0.1 |
| DNS | 10.0.0.1 |

| ## String / Data Registers |  |  |  |  |
| --- | --- | --- | --- | --- |
| ## Address | ## Name | ## FC | ## Max registers | ## Description |
| ## 30 | ## TEXT | ## 16 | ## 30 (60 bytes) | ## Custom message string (null-terminated, packed as register pairs) |
| ## 60 | ## IVAR | ## 16 | ## 10 | ## Integer variable display (appends SI unit suffix) |
| ## 80 | ## WFVAR | ## 16 | ## 2 | ## Write floating-point value (4 bytes per register pair) |
| ## 90 | ## RFVAR | ## 3 | ## 2 | ## Read floating-point value (read-only) |

190mm Dimensions

172mm 166.5mm 180mm 28.6mm O 100mm 35mm 124mm 62mm

| ## Multi-Register Grouped Writes — FC16 |  |  |  |
| --- | --- | --- | --- |
| ## Address | ## Qty | ## Function | ## Registers written (in order) |
| ## 0 | ## 4 | ## Set XY geometry | ## POSX, POSY, ENDX, ENDY |
| ## 4 | ## 4 | ## Set shape | ## SHAPE, SHAPE_Q, SHAPE_WEIGHT, SHAPE_COLOR |
| ## 8 | ## 2 | ## Set bitmap | ## BITMAP, BITMAP_Q |
| ## 10 | ## 2 | ## Set border | ## BORDER_WEIGHT, BORDER_COLOR |
| ## 12 | ## 3 | ## Set text properties | ## TEXT_POSX, TEXT_POSY, TEXT_COLOR |
| ## 24 | ## 2 | ## Set scroll | ## ROTDIR, ROTSPEED |
| ## 26 | ## 2 | ## Fill screen | ## FILL_SCREEN, FILL_COLOR |

Installation

LED-O can be connected to the Ethernet port of a PLC, then supplied with 24v DC

DC 24V CPU PLC DC INPUT COM1 OUTPUT POWER SUPPLY